Optimization of the Design of Labyrinth Weirs

# Abstract

Labyrinth weirs are increasingly common in dam rehabilitation projects due to their passive ability to pass large discharges at low heads. Their complex geometry presents designers with multiple configurations that can produce the same capacity. This study presents a multi objective nonlinear program that seeks to simplify the design of labyrinth weirs. This is accomplished by using industry standard design methods to calculate geometry based on hydraulic/site inputs by the designer. The program presents a pareto optimal front that maximizes efficiency while minimizing the volume of concrete (cost). This pareto optimal front can better inform designers on the different possible configurations and decide which tradeoffs should be made.

# Introduction

Weirs are important structures for water supply that help control the release of flow. Labyrinth weirs are a linear weir that is folded like an accordion perpendicular to the flow of water to create a longer crest length and subsequently pass more flow. Labyrinth weirs allow for larger discharges to occur at smaller upstream heads. Labyrinth weirs have grown in popularity as dam rehabilitation occurs across the world. They are a favorable choice because they allow for passive control while still passing needed flows set by regulators. There has been much study in the last couple of decades into labyrinth weirs and their complicated geometry.

The goal of this project is to create a program that will optimize the design of labyrinth weirs. Labyrinth weir geometry requires the designer to select many different characteristics to calculate the shape and weir length which ultimately determines the capacity of the weir. With so many variables that can be changed and yet still produce the same capacity it can be hard to know which configuration to go with. Cost of construction is what often governs the decision of which configuration to use. This project seeks to balance the cost of construction with an efficiency parameter found by Crookston and Tullis (2012). The goal of the program will be to maximize efficiency and minimize the cost of construction (volume of concrete).

Literature Review

There has been much research into labyrinth spillways in the last two decades to help improve the design and methods of their implementation. P Tullis, Manian and Waldron (1995) published one of the original design reference documents which established a methodology for how to design Labyrinth weirs. Crookston and B Tullis (2012a) further expanded on previous work by publishing a paper that outlined further how to design a Labyrinth weir with clear methodology and also with published Cd equations and values. Using physical models, they examined weir geometries with side walls angles between 6 and 35 degrees. This paper also produced the parameter of efficiency which quantifies how well flow passes over the weir and how water interacts with itself as it passes over different cycles with different geometries.

Crookston and B Tullis (2012b) also looked at the impacts of aeration and nappe instability and the effects those have on discharge. This was an important study that looked deeper into some of the complications that exist with labyrinth weir geometries. Aeration and submergence are not covered in the model described in this report but should be considered by the designer given the results of the model. Dabling, Tullis, and Crookston (2012) further examined labyrinth weirs, more specifically looking at having stages in the weir that allow smaller flows to be passed at lower heads. This work allows for more flexibility in design of a labyrinth that must pass a range of flows. The program described in this report does not cover staged labyrinth weirs due to their complexity.

More recently Mahmoud et al. (2021) took the data produced in 2012 and sought to create a neural network to optimize the design of labyrinth weirs. Their study built a complicated neural network that would interpolate discharge coefficients calibrated using previous data. The model described in this report seeks to do something similar to the work of Mahmoud et al. but in a simpler manner that could be understood and interpreted by designers and engineers outside the world of academia.

# Methods

This section outlines the model formulation and methods used to solve this problem. Figure 1 provides a graphic of how what each variable means and how it plays into the larger design.

Figure 1. Crookston & Tullis 2012

## Decision Variables

α – Side wall angle. The angle from the centerline of each cycle to the walls of the cycle. (6, 8, 10, 12, 15, 20, 35) [deg]

B – Length of apron, the length from the upstream point of the apex to the downstream point of the apex. [len]

N – Number of cycles. [-]

w – Cycle width. The width between the center of each apex in a cycle. [len]

P – Crest height. The total height of the weir from the floor of the apron. [len]

A – Inside apex width. The inside dimension between the angled side walls in each cycle. [len]

## State Variables

W – Total width of floor for the entire width. [len]

W=w*N#1

lc – Centerline length of sidewall. [len]

lc=B-twcosα #2

Lc – Total centerline length of weir. [len]

Lc=N*2*A+twtan45-α2+lc #3

Cd – Labyrinth weir discharge coefficient. This will be a function based on data collected by (Crookston & Tullis 2012) [-]

Cdα0=fHTP, α, Crest shape#(4)

M – Magnification ratio. A unitless ratio of length of sidewalls to the width and number of cycles. [-]

M=lcwN#5

h – Head of water surface elevation above the weir height [len]

h=H-P#6

HT – Total upstream head. Piezometric head plus velocity head [len]

HT=h+v22g#7

Q – Capacity of the Weir [len/s]

Q=23cdα0Lc2gHT32#8

## Objective Functions

Minimize volume of concrete [len3]

V=L*P*tw+W*B*tf#9

Maximize efficiency [-]

ε=Cdα0MCd900#10

## Input Conditions (Designer input constraints)

H – Design flow water surface elevation. The desired water surface elevation of the reservoir [len]

Hapron – Lowest Elevation for apron floor. The lowest elevation the floor can be which will determine the height of weir and head. [len]

Wmax – The max width of floor that the weir can take. [len]

Bmax – The max length of the apron [len]

tf – Thickness of the weir foundation [len]

tw – Thickness of weir wall at the crest. [len]

Vapproach – Reservoir approach velocity [len/s]

Qdesign – Design Flowrate, the minimum capacity of the weir [len3/s]

## Constraints

w/P - Cycle width ratio (2≤w/P≤4)

A/w – Apex Ratio (A/w<0.08)

P/tw - Relative thickness ratio (7.5 ≤Ptw≤8.5)

HT/P – Total Head to Weir Height Ratio [Crookston & Tullis 2012 Data restriction] (0.05 ≤HTP ≤0.9)

Q – The calculated flowrate must be equal to or greater than Qdesign (Q≥Qdesign)

Figure 2. Crookston & Tullis 2012 Cd Values

# Solving

To use this program the designer must first input certain hydraulic inputs listed above. These inputs are site specific conditions that constrain the model. These parameters are typical constraints common to any situation where a labyrinth weir is being considered. A designer may modify these values to have more control over the model.

This problem was solved using GAMS v51 software to find the pareto optimal front of this multi objective function. Given that there are two objectives to this problem it remains the designers decision to pick which objective they prioritize cost or efficiency. Gvien the non linear nature of several equations in this model such as the Cd equation a non linear program must be used. The full GAMS code can be found at this repository, GitHub (https://github.com/SpenYoung/CEE-6410-Young/tree/3e9f9bc160023f0aef61572e40c5cbad8c5ec3b0/Final%20Project).

To find the pareto optimal front a maximum efficiency and subsequent volume of concrete is first found to have a basis for the other points. From there a loop is used to find the volume of concrete at different percentages of the maximum efficiency, by changing the geometry variables. The model then produces a csv file containing the different efficiency and volume variables as well as the geometry variables associated with each scenario. This allows the designer to complete checks to make sure the model is working correctly and producing valid results, but also allows for a pareto optimal front to be graphed and then used for decision making for the final design. An example of a produced pareto optimal front can be seen in Figure 3.

Figure 3. Example of a Pareto Optimal front Generated by the Model

# Conclusion

This report presents a program to serve as a framework for the design of labyrinth weirs. The program seeks to present the pareto optimal front created by minimizing the volume of concrete and maximizing efficiency while meeting given site constraints and hydraulic requirements. The program finds the optimal geometry to meet capacity requirements while trading off benefits of volume or efficiency.

The model is implemented in GAMS as a multi objective nonlinear program. The model allows the designer to input given site and hydraulic data and then produces a pareto optimal front that allows the designer to visualize tradeoffs between the objective functions. This program seeks to be transparent and easy to manipulate by being in the simple coding language of GAMS without requiring neural networks or complicated algorithms. The model allows the user to export a .csv file after calculations have been made which include any geometry parameters the user would like. These values can then be checked in traditional spreadsheet calculations to ensure the accuracy of the design.

It is important to note the limitations of this model. This model does not check for submergence; this should be checked by the designer after getting the geometry from the model. The model also does not check for nappe aeration, which is another parameter that should be checked outside the model by the designer. This model is also constrained to only be a single stage labyrinth weir and if the designer is seeking a staged labyrinth weir they should use other calculation methods.

# Appendix A - Annotated Bibliography

Design of Labyrinth Spillways (1994)

J. Paul Tullis, Nosratollah Amanian & David Waldron

https://ascelibrary-org.dist.lib.usu.edu/doi/abs/10.1061/(ASCE)0733-9429(1995)121:3(247)

This paper published by Tullis et al. is one of the first cohesive design documents for labyrinth spillways. It created a clear procedure for how these structures could be designed. A framework was provided with detailed steps to follow for any designer. They used extensive physical modeling to create Cd values for sidewall angles of 6deg to 35deg. They also produced a spreadsheet that could be used by designers to streamline the calculations for labyrinth weirs.

Hydraulic Design and Analysis of Labyringth Weirs. I: Discharge Relationships (2012)

B. M. Crookston & B. P. Tullis (2012)

https://ascelibrary-org.dist.lib.usu.edu/doi/epdf/10.1061/%28ASCE%29IR.1943-4774.0000558

Crookston and Tullis built upon previously completed studies of labyrinth weirs using physical modeling at the Utah Water Research Laboratory. The aim of their work was to establish more discharge coefficients for sidewall angles of 6deg to 35deg. They also researched different crest geometries such as quarter-round and half-round shapes. Their work clearly maps out a method for how to design a labyrinth weir with definitions for each input variable. Using physical modeling they were able to collect an entire data set of coefficient values (Cd) for different head to weir height relationships. Curves were then fit to the data the coefficients of the fit lines are published to be used in practice.

Hydraulic Design and Analysis of Labyrinth Weirs. II: Nappe Aeration, Instability, and Vibration (2012)

B. M. Crookston & B. P. Tullis

https://ascelibrary-org.dist.lib.usu.edu/doi/10.1061/%28ASCE%29IR.1943-4774.0000553

Crookston and Tullis expanded upon their previous work with labyrinth weirs by examining other qualities and scenarios that may affect efficiency. The nappe and need for aeration has been studied for other weir geometries, but this paper expounds upon that work by investigating those characteristics on labyrinth weirs. Through more physical model testing conducted at the Utah Water Research Laboratory it was found that there are four different nappe conditions based on headwater to weir height ratios and side wall angles. The four conditions are clingin, aerated, partially aerated, and drowned. It was observed that a clinging nappe has the most efficiency. A phenomenon called nappe instability was also observed and documented for under what conditions this phenomenon occurred. The goal of the paper is to encourage designers to consider the nappe and aeration in their designs and how it may affect the efficiency of their designs.

Staged Labyrinth Weir Hydraulics (2013)

M. R. Dabling, B. P. Tullis, & B. M. Crookston

https://ascelibrary-org.dist.lib.usu.edu/doi/10.1061/%28ASCE%29IR.1943-4774.0000636

This paper further developed the work completed by Tullis and Crookston in 2012 but looked at a different design of labyrinth weirs with staged elevations. The staged elevations approach allows for flow to begin passing over the weir at lower water surface elevations without having to activate the entire weir. This allows for staged releases giving more control in a passive structure. Laboratory testing was completed at the UWRL and involved testing multiple staged configurations with differing parameters such as height offset, location of offset, and segment lengths. A method of calculating the total discharge was given by summing the discharge contribution of each cycle of the weir. Coefficients are published in the paper to help aid those who want to implement stages into their labyrinth weir design

Investigation on labyrinth spillway multi-objective optimization with an emphasis on predicting discharge coefficient through different artificial neural networks

Ali Mahmoud et al.

https://www-sciencedirect-com.dist.lib.usu.edu/science/article/pii/S0263224121000683?ref=pdf_download&fr=RR-2&rr=98e8b2814aec2b2a

This paper builds upon the work previously completed by Crookston and Tullis and sought to interpolate Cd values and optimize design by maximizing capacity and minimizing concrete. This group took the data published in 2012 by Crookston and Tullis and used neural networks to interpolate Cd values for angles between those previously studied. To accomplish this, they used several different neural network strategies including MLP, SVR, and RBFNN. It was found that the Multi-Layer Perceptron (MLP) performed the best for interpolating Cd values, compared to the data collected in a laboratory. They then used these interpolated values to complete a redesign of a labyrinth weir that is in place at Ute dam. The solution they created passed the same capacity and reduced concrete by 21.47%.

# Appendix B – Comments Addressed

Comments addressed from Dr. Rosenberg’s review:

- No abstract Present

  - Abstract added at the beginning of the report

- Not properly organized

  - Now organized with abstract, introduction, literature review, methods, and conclusion

- No conclusion Present

  - Conclusion added at the end of the report

- No original tables/figures

  - Figure 3 added as an output from the model

- Equations not numbered

  - Equations have been numbered

- Literature is only an annotated bibliography and not comprehensive

  - A more comprehensive literature review has been written after the introduction with annotated bibliography moved to Appendix A

  - While there are many journals by Crookston and Tullis this is because their contributions to labyrinth weir designs has been very influential and much design is still completed based on their results. Their work is good primary sources.

Comments addressed from Tyler Mindrum’s review:

- No abstract Present

  - Abstract added at the beginning of the report

- Not properly organized

  - Now organized with abstract, introduction, literature review, methods, and conclusion

- No conclusion Present

  - Conclusion added at the end of the report

- Figure 1 not referneced

  - Figure 1 now reference in methods section

- Equations not numbered

  - Equations have been numbered

Comments addressed from Jacob Ramsey’s review:

- Not properly organized

  - Now organized with abstract, introduction, literature review, methods, and conclusion

- No clear conclusion Present

  - Conclusion added at the end of the report

- Equations not numbered

  - Equations have been numbered

| Category (Possible Score) | No Evidence | Does not Meet Standard | Nearly Meets Standard | Meets Standard | Exceeds Standard | Self- Score | Instructor Score |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Title (2) | Absent 0 | Evidence of one. 0 | Evidence of two. 1 | Evidence of three. 1 | Title, author name and contact info. Neatly finished with no errors. 2 | 2 |  |
| Introduction (10) | Absent, no evidence 0 | There is no clear introduction, main topic, or outline of content. 1 - 5 | The introduction is either: - Too sketchy. Gives an inadequate overview, Or: - Too detailed, info later repeated 6 – 7 | The introduction overviews the project and previews the wiki page(s) structure 8 | The introduction overviews the project, work done, and organization of the wiki page(s). An effective summary. Gives enough detail to motivate the reader to continue reading. 9 - 10 | 10 |  |
| Technical Content (43) | No content provided or analysis evident. 0 - 10 | Little content provided. The reader has no idea about the problem, solution method, results, or what was done for the project. 11 – 20 | Sketchy: may have left out 2 or more content areas; flimsy or incomplete methods; results have errors; and/or recommendations do not derive from the results. No tables, figures, or pictures presented. 21 – 33 | Wiki lacks adequate detail, but content for 4 of the 5 areas is provided and includes one or more tables, figures, or pictures. Most prior work referenced and hyperlinked. 34 – 38 | Defines problem, provides background information, describes solution method(s) used, and presents the results and recommendations that derive from the results. Uses tables, figures, and pictures to illustrate the above. Prior work referred to through references and hyperlinks. 39 - 43 | 40 |  |
| Organization and Development (15) | No evidence of structure. | Little evidence of structure or organization. 1 – 8 | Organization of ideas not fully developed. Two or more pages, sections, or sub-sections missing or out of order. 9 – 11 | Sub-pages, sections, sub-sections, and/or lists present, but their use not perfected. 12-13 | Logical sequencing of ideas. Uses sub-pages, sections, sub-sections, and/or lists to order, present, and develop ideas. In each section, one or more paragraphs develop each idea. 14 - 15 | 15 |  |
| Category (Possible Score) | No Evidence | Does not Meet Standard | Nearly Meets Standard | Meets Standard | Exceeds Standard | Self- Score | Instructor Score |
| Word Usage and Format (15) | Not applicable | Many, distracting errors in grammar, spelling, sentence structure, word usage, significant figures, tables, and figures. Unacceptable at the graduate level. 1 – 8 | With some grammatical errors. Figures are too small and/or under-labeled, although they are usually of acceptable quality and focus. Incoherent tables. Inconsistent fonts and headings. Could be improved by being more meticulous. 9 – 11 | Almost no errors in punctuation, capitalization, spelling, sentence structure, word usage, significant figures, and presentation of figures and tables. No broken hyperlinks.12–13 | Punctuation, capitalization, spelling, sentence structure, word usage, and significant figures all correct. Clear, consistent fonts and headings. Good wiki processing skills. Figures and tables presented in correct format. No broken or empty hyperlinks. 14 - 15 | 15 |  |
| Conclusions (10) | Absent 0 | Incomplete and/or not focused. 4 - 6 | The conclusion does not adequately restate the main findings. 7 | The conclusion restates the main findings. 8 | Effectively restates the main findings and recommendations to solve the problem. 9 - 10 | 10 |  |
| Hyperlinks and References (5) | Absent 0 | With many errors or only 1 hyperlink provided. 1 – 2 | With some errors and only 2 hyperlinks provided. 3 | With few errors, at least 3 hyperlinks to content outside the USU domain 4 | All citations and references listed in ASCE format with no errors. Include at least 4 hyperlinks to content or work outside the USU domain. 5 | 5 |  |
| TOTAL (100) |  |  |  |  |  |  |  |